Proxiant Academy

AI for Operations

Class 2

Operational Data Engineering & Analytics

Week 2

Learning Objectives

- Understand IoT sensor types and sampling requirements for operations

- Design real-time data pipelines: MQTT → Kafka → Stream Processing → Storage

- Integrate ERP systems (SAP, Oracle, WMS, TMS) with analytics infrastructure

- Master time-series database design with InfluxDB and TimescaleDB

- Engineer features from raw sensor data for ML modeling

Detailed Topic Coverage

IoT Sensors for Operations

Vibration Sensors (Accelerometers)

- Measure mechanical vibration to detect bearing wear, imbalance, misalignment

- Sampling: 10kHz typical for high-frequency detection

- Output: RMS (root mean square), peak frequency, kurtosis

- Example sensor: ICP accelerometer IEPE (±5V, 100mV/g sensitivity)

import numpy as np
accel_data = np.random.randn(100000)  # 10Hz × 10 seconds
rms = np.sqrt(np.mean(accel_data**2))

Temperature Sensors (Thermocouples)

- Monitor equipment surface temperature, cooling system health

- Type K thermocouple: -200°C to +1250°C, cost $5-20

- Sampling: 1Hz typical (slow thermal changes)

- Alerts: Threshold monitoring, rate-of-change detection

Pressure Transducers

- Track pneumatic/hydraulic system health, gas/liquid flow

- Range: 0-250 bar typical, 4-20mA signal

- Sampling: 100Hz for high-pressure systems

Flow Meters

- Measure liquid/gas flow rates in pipelines

- Pulse-based: Each pulse = fixed volume (0.1L per pulse)

- Sampling: Event-driven or 1Hz aggregation

Real-Time Data Pipeline

Architecture: MQTT → Kafka → Flink → Storage

- MQTT Broker (Mosquitto): Device publish at /sensors/facility/line1/vibration

- Kafka Topics: By sensor type (sensors-vibration, sensors-temp) with 3-day retention

- Flink Jobs: Parse AVRO, validate, feature engineering in 1-second windows

- Storage: Hot (InfluxDB 7-day) + Cold (S3 parquet, 2-year retention)

# Python Kafka producer example
from kafka import KafkaProducer
producer = KafkaProducer(bootstrap_servers=['localhost:9092'])
producer.send('sensors-vibration', sensor_data_json)

ERP Integration

SAP RFC Connectors

- Remote Function Call (RFC) protocol for SAP systems

- Pull production orders (SAP MFGORD), schedule changes in real-time

- Example: Query module MM_STOCK for current inventory by warehouse

pip install pyrfc
# Connect to SAP system
from pyrfc import Connection

Oracle REST APIs

- Oracle ERP Cloud exposes REST endpoints for SO (sales order), PO (purchase order), inventory

- Authentication: OAuth2 token

import requests
headers = {'Authorization': f'Bearer {token}'}
resp = requests.get('https://ebs.oracle.com/api/orders', headers=headers)

WMS (Warehouse Management) Events

- Receive/pack/ship events logged in real-time

- Kafka topic: wms-events with timestamps, product SKU, location, quantity

- Use: Calculate throughput KPIs, identify bottlenecks

Time-Series Databases

InfluxDB with Flux Query Language

- Schema-less: tag=device_id,location and field=temperature value

- Retention: 7-day hot cache, older data moved to S3/cold storage

- Flux query: Filter by time range, aggregate 5-minute averages

from influxdb import InfluxDBClient
client = InfluxDBClient(host='localhost', port=8086)
points = [{'measurement': 'vibration', 'fields': {'rms': 0.45}, 'time': now}]
client.write_points(points)

TimescaleDB Hypertables

- PostgreSQL extension for time-series at scale (100B+ data points)

- Automatic time-based partitioning (chunks), compression

- Continuous aggregates: Pre-computed 1-hour roll-ups

CREATE TABLE sensor_data (time timestamptz, device_id int, value float);
SELECT create_hypertable('sensor_data', 'time');

Data Quality & Preprocessing

Sensor Drift Calibration

- Vibration sensor gain changes over time

- Solution: Periodic baseline calibration, track drift over days

- Adjust: raw_value = (measured - drift) / gain

Missing Value Imputation

- Linear interpolation: Connect two points with line

- Spline interpolation: Smooth cubic curves

- Forward-fill: Repeat last value (for ~30 second gaps)

df['temp'].interpolate(method='linear', limit_direction='both')

Outlier Detection (Z-Score)

- Flag values > 3σ from mean as anomalies

- Exclude from training, investigate root cause

z_scores = np.abs((X - X.mean()) / X.std())
outliers = z_scores > 3

Feature Engineering from Sensors

Time-Window Aggregation

- Rolling mean (1min, 5min, 1hr windows): Smooth short-term noise

- Rolling std: Measure variability (high std = unstable process)

- Rolling max/min: Detect peaks and valleys

df['vibration_ma_5min'] = df['vibration'].rolling('5min').mean()

Lag Features

- Previous values: lag_1 (1 minute ago), lag_7 (7 minutes ago)

- Helps model learn auto-correlation in time series

df['temp_lag_1'] = df['temp'].shift(1)

FFT (Fast Fourier Transform) for Seasonality

- Decompose vibration signal into frequency components

- Extract dominant frequencies (e.g., 60Hz = electrical)

from scipy.fft import fft
frequencies = fft(vibration_signal)

Domain Features

- Cycle count: Number of machine cycles since start

- Throughput: Items processed per minute

- Equipment utilization: Uptime / scheduled hours

Hands-On Lab: Sensor Data Pipeline

Step 1: Install Kafka & InfluxDB

docker pull confluentinc/cp-kafka:latest
docker pull influxdb:latest
docker-compose up -d

Step 2: Create Kafka Producer (Python)

pip install kafka-python
from kafka import KafkaProducer
import json, time
producer = KafkaProducer(bootstrap_servers=['localhost:9092'], value_serializer=lambda v: json.dumps(v).encode())
for i in range(100):
    producer.send('sensors', {'device': 'pump1', 'temp': 65 + i*0.1, 'ts': time.time()})

Step 3: Configure InfluxDB Write

from influxdb_client import InfluxDBClient, Point
client = InfluxDBClient(url='http://localhost:8086', token='YOUR_TOKEN', org='proxiant')
write_api = client.write_api()
point = Point('sensor_data').tag('device', 'pump1').field('temp', 65.5)
write_api.write(bucket='sensors', record=point)

Step 4: Build Grafana Dashboard

- Connect Grafana to InfluxDB data source

- Create panel: SELECT mean("temp") FROM sensor_data WHERE time > now() - 7d GROUP BY time(1m)

- Set alerts: If temp > 80°C for 5 minutes, trigger webhook

Key Concepts Summary

| Concept | Details | Use Case |
| --- | --- | --- |
| MQTT | Lightweight pub/sub protocol, 10-100KB messages | Device → Broker communication |
| Kafka | Distributed streaming platform, high throughput | Multi-source data aggregation |
| InfluxDB | Time-series database with retention policies | Hot storage for analytics |
| TimescaleDB | PostgreSQL extension with 100B+ datapoint scale | Long-term time-series |
| Hypertable | Auto-partitioned time-series table | Efficient range queries |
| Feature Engineering | Transform raw sensor to ML features | Rolling stats, FFT, lags |
| Z-Score | Standard deviation outlier detection | Data quality validation |
| Continuous Aggregates | Pre-computed materialized views | Real-time dashboard KPIs |
| RFC Protocol | SAP remote function call | ERP system integration |
| WMS Events | Warehouse pick/pack/ship log stream | Throughput monitoring |

Code Examples

Complete Sensor Data Pipeline

import pandas as pd, numpy as np
from kafka import KafkaConsumer
df = pd.DataFrame(consumer_messages)
df['rms'] = df['vibration'].rolling(100).apply(lambda x: np.sqrt(np.mean(x**2)))
df['is_anomaly'] = np.abs((df['rms'] - df['rms'].mean()) / df['rms'].std()) > 3

References

- Apache Kafka Documentation: https://kafka.apache.org/docs/

- InfluxDB Flux: https://docs.influxdata.com/influxdb/latest/query-data/get-started/

- TimescaleDB: https://docs.timescale.com/

- SAP RFC: https://www.sap.com/

Next Class Preview

Class 3: Demand Forecasting with ML

- ARIMA & exponential smoothing for univariate forecasting

- XGBoost/LightGBM with multi-source features

- Deep learning: LSTM, N-BEATS, Temporal Fusion Transformer

- Hierarchical forecasting and reconciliation

- Probabilistic forecasting: quantile regression, safety stock