Proxiant Academy

AI for Operations

Class 6

Warehouse Automation & Robotics

Week 6

Learning Objectives

- Implement ABC slotting optimization: velocity-based zone assignment for fast picking

- Solve pick-path problem: TSP formulation, nearest-neighbor heuristic, 2-opt improvement

- Evaluate robotics technologies: AMRs, goods-to-person, cobots, computer vision

- Build digital twin: SimPy discrete event simulation with real-time IoT feed

- Optimize warehouse layout and operations with data-driven insights

Detailed Topic Coverage

ABC Slotting Optimization

ABC Analysis Framework

- A items: Top 20% of SKUs, 80% of revenue (fast movers)

- B items: Middle 30% of SKUs, 15% of revenue (medium)

- C items: Bottom 50% of SKUs, 5% of revenue (slow movers)

- Example: 10,000 SKUs → 2,000 (A), 3,000 (B), 5,000 (C)

import pandas as pd
sku_sales = pd.DataFrame({'sku': skus, 'units_sold': sales})
sku_sales = sku_sales.sort_values('units_sold', ascending=False)
sku_sales['cum_pct'] = sku_sales['units_sold'].cumsum() / sku_sales['units_sold'].sum()
sku_sales['class'] = sku_sales['cum_pct'].apply(lambda x: 'A' if x <= 0.8 else ('B' if x <= 0.95 else 'C'))

Velocity-Based Zone Assignment

- Zone 1 (fast pick): A items, closest to packing, waist height

- Zone 2 (medium pick): B items, moderate distance, shoulder height

- Zone 3 (slow pick): C items, furthest, floor/overhead

- Impact: Picker travel distance -30%, throughput +25%

ML-Predicted Demand for Dynamic Slotting

- Forecast next month demand for each SKU

- Re-assign locations monthly based on predicted velocity

- Python: Train LightGBM on historical demand, predict next month

Pick-Path Optimization

TSP Formulation

- Traveling Salesman Problem: Find shortest path visiting all items once

- Decision: x_ij = 1 if picker travels from item i to item j

- Objective: Minimize total distance

- NP-hard: Heuristics for 100+ items

Nearest-Neighbor Heuristic

- Start at depot, visit nearest unvisited item, repeat

- Time: O(n²), solution quality 125% of optimal

def nearest_neighbor(locations):
  route = [0]  # start at depot
  unvisited = set(range(1, len(locations)))
  while unvisited:
    current = route[-1]
    nearest = min(unvisited, key=lambda i: distance(locations[current], locations[i]))
    route.append(nearest)
    unvisited.remove(nearest)
  return route

2-Opt Local Search Improvement

- Iteratively swap edges: if reversing segment reduces distance, keep swap

- Time: O(n²) per iteration, 5-10 iterations typical

- Improvement: 125% → 105% of optimal

def two_opt(route, locations):
  improved = True
  while improved:
    improved = False
    for i in range(1, len(route)-2):
      for j in range(i+1, len(route)):
        new_route = route[:i] + route[i:j][::-1] + route[j:]
        if total_distance(new_route) < total_distance(route):
          route = new_route
          improved = True
  return route

Batch Picking Strategy

- Combine orders by zone: reduce duplicate visits

- Sort by aisle: walk each aisle once, pick all items

- Time: Single order 20 minutes, 5-item batch 25 minutes (80% efficiency)

Robotics in Warehousing

Autonomous Mobile Robots (AMRs)

- Examples: Locus Robotics, 6 River Systems (acquired by Shopify)

- Capability: Goods-to-person delivery, 100-300kg payload, 1.5 m/s speed

- Fleet size: 200-vehicle system handles 3-4x throughput of manual

- Cost: $25-40K per robot, payback 2-3 years

Amazon Kiva Robots (Now Housed by Amazon)

- Performance: 300+ robots per fulfillment center

- Task: Transport bin from storage to human picker

- Impact: Picker stays at station, 50% time reduction, 600 items/hour

Collaborative Robots (Cobots)

- Universal Robots UR5/UR10: 5-10kg payload, human-safe

- Application: Depalletizing, piece-picking from complex bins

- Speed: 30-50 picks/minute (vs 40-60 manual)

Automated Storage/Retrieval Systems (AS/RS)

- Carousel or vertical towers: 95% space utilization

- Throughput: 300-600 retrievals/hour

- Cost: $500K-2M per installation

Computer Vision in Warehousing

Barcode/QR Code Scanning

- Library: OpenCV (Python), barcode detection

- Accuracy: 99.5% read rate at 20cm distance

import cv2
from pyzbar.pyzbar import decode
img = cv2.imread('barcode.jpg')
barcodes = decode(img)
for barcode in barcodes:
  print(barcode.data)

Damage Detection CNN

- Binary classifier: damaged (crease, dent, stain) vs intact

- Training: 5K labeled images, ResNet50 backbone

- Accuracy: 96% on test set

from tensorflow.keras.applications import ResNet50
from tensorflow.keras.layers import Dense, GlobalAveragePooling2D
base_model = ResNet50(weights='imagenet', include_top=False)
x = GlobalAveragePooling2D()(base_model.output)
output = Dense(1, activation='sigmoid')(x)

Inventory Counting with Drone + YOLO

- Drone equipped with RGB camera, flies grid pattern

- YOLO object detector identifies items in view

- Speed: Count 10K items in 30 minutes (vs 2 hours manual)

Dimensioning with Intel RealSense

- Depth camera measures height/width/length of parcel

- Accuracy: ±5mm, real-time processing

- Use: Identify oversized items, optimize packing

Digital Twin & SimPy Simulation

Digital Twin Definition

- 3D virtual model of warehouse + real-time IoT feed

- Synchronize: Robot locations, item locations, picker positions

- Enable: What-if scenarios, predictive analytics

# 3D model loaded from CAD, updated with real-time positions
warehouse_3d = load_warehouse_model('warehouse.glb')
for robot in robot_locations:
  update_position(warehouse_3d, robot.id, robot.gps)

SimPy Discrete Event Simulation

- Simulate warehouse operations: orders arrive, pickers pick, robots deliver

- Metrics: throughput (picks/hour), utilization (%), average order time

import simpy
def picker(env, warehouse):
  while True:
    order = yield env.process(warehouse.get_order())
    yield env.timeout(order.pick_time)
    print(f"Order {order.id} picked at {env.now}")

What-If Scenarios

- Scenario 1: Add 10 new robots → throughput +15%

- Scenario 2: Change layout to reduce picking distance 20% → throughput +12%

- Scenario 3: Seasonal surge (50% demand increase) → need 5 temp workers

Hands-On Lab: Pick-Path Optimizer

Step 1: Define Warehouse Layout

import numpy as np
# 10x10 warehouse grid
grid = np.zeros((10, 10))
depot = (0, 0)
item_locations = [(2, 3), (5, 7), (8, 1), (3, 9), (7, 5), (1, 6), (9, 8), (4, 4), (6, 2), (8, 9)]

Step 2: Implement Euclidean Distance

def distance(p1, p2):
  return ((p1[0]-p2[0])**2 + (p1[1]-p2[1])**2)**0.5
def total_distance(route, locations):
  dist = 0
  for i in range(len(route)-1):
    dist += distance(locations[route[i]], locations[route[i+1]])
  return dist

Step 3: Nearest-Neighbor Heuristic

def nearest_neighbor_route(locations):
  route = [0]
  unvisited = set(range(1, len(locations)))
  while unvisited:
    current = route[-1]
    nearest = min(unvisited, key=lambda i: distance(locations[current], locations[i]))
    route.append(nearest)
    unvisited.remove(nearest)
  route.append(0)  # return to depot
  return route

Step 4: 2-Opt Improvement

def two_opt_improve(route, locations, max_iter=10):
  for _ in range(max_iter):
    improved = False
    for i in range(1, len(route)-2):
      for j in range(i+1, len(route)-1):
        new_route = route[:i] + route[i:j][::-1] + route[j:]
        if total_distance(new_route, locations) < total_distance(route, locations):
          route = new_route
          improved = True
  return route

Step 5: Compare and Visualize

naive_route = list(range(10))  # order 0-9
nn_route = nearest_neighbor_route(item_locations)
optimized = two_opt_improve(nn_route, item_locations)
print(f"Naive: {total_distance(naive_route, item_locations):.1f}")
print(f"NN: {total_distance(nn_route, item_locations):.1f}")
print(f"Optimized: {total_distance(optimized, item_locations):.1f}")

Key Concepts Summary

| Concept | Description | Impact |
| --- | --- | --- |
| ABC Slotting | Zone assignment by velocity (A=fast, B=med, C=slow) | Picking distance -30% |
| TSP | Traveling Salesman: shortest path visiting all items | Foundational for pick optimization |
| Nearest-Neighbor | Greedy heuristic, O(n²), quality 125% optimal | Quick approximate solution |
| 2-Opt Local Search | Swap edges iteratively to improve route | Quality 105% optimal, 5-10 iterations |
| AMR | Autonomous Mobile Robot for goods-to-person | 200 units/center, 2-3 year payback |
| Kiva Robot | Goods-to-person bin carrier, 300+ per center | 50% picker time reduction |
| Cobot | Collaborative robot (UR5/UR10) for piece-picking | Safe human-robot interaction |
| AS/RS | Automated Storage/Retrieval System (carousel/tower) | 300-600 picks/hr, 95% space util |
| YOLO | Real-time object detection CNN | Inventory count drone: 10K items/30min |
| Digital Twin | Virtual 3D + real-time IoT, what-if scenarios | Simulate operations, optimize layout |

Code Examples

Complete Pick-Path Flow

all_locations = [(0, 0)] + item_locations  # depot + items
route = nearest_neighbor_route(all_locations)
route = two_opt_improve(route, all_locations)
savings = total_distance(list(range(11)), all_locations) - total_distance(route, all_locations)
print(f"Route: {route}")
print(f"Distance savings: {savings:.1f} meters")

References

- SimPy Discrete Event Simulation: https://simpy.readthedocs.io/

- OpenCV Computer Vision: https://opencv.org/

- YOLO Object Detection: https://github.com/ultralytics/yolov5

- Warehouse Automation Survey: Boysen et al. (2019)

Course Conclusion

Course Summary

- Class 1: AI/ML taxonomy, industry impact, case studies, Python setup

- Class 2: IoT sensors, real-time data pipelines, ERP integration, feature engineering

- Class 3: Demand forecasting with ARIMA, XGBoost, LSTM, probabilistic methods

- Class 4: Facility location, inventory optimization, supplier scoring, bullwhip effect

- Class 5: Vehicle routing with OR-Tools, real-time GPS, fleet telematics, maintenance prediction

- Class 6: Warehouse slotting, pick-path optimization, robotics, computer vision, digital twins

Key Takeaways

- AI/ML is not one-size-fits-all: choose methods by problem type (classification, regression, optimization)

- Data quality drives results: sensor calibration, ERP integration, feature engineering critical

- Operations is constrained optimization: capacity, time windows, service levels shape solutions

- Real-time integration matters: GPS tracking, weather, traffic, demand signals require adaptive systems

- ROI focus: measure labor savings, distance reduction, throughput improvement in business terms

Recommended Projects

- 1. Build demand forecast model with 2+ SKUs, evaluate MAPE vs seasonal baseline

- 2. Formulate and solve facility location LP for 50+ locations with PuLP

- 3. Implement VRP solver for 20-customer routing problem with capacity/time constraints

- 4. Design pick-path algorithm for 100-item warehouse with 2-opt optimization

- 5. Create digital twin simulation with SimPy, test 3+ operational scenarios

Additional Resources

- Simchi-Levi, D., Kaminsky, P., Simchi-Levi, E. (2008). Designing and Managing the Supply Chain. McGraw Hill.

- Goodfellow, I., Bengio, Y., Courville, A. (2016). Deep Learning. MIT Press.

- Google OR-Tools: https://developers.google.com/optimization

- scikit-learn Documentation: https://scikit-learn.org/

- TensorFlow/Keras: https://www.tensorflow.org/