Proxiant Academy

AI for Operations

Predictive Maintenance with AI

Class 7

Confidential

Learning Objectives

Learning Objectives

- Vibration analysis FFT

- RUL prediction survival

- Anomaly CUSUM/EWMA

- P-F curves

- Maintenance cost models

- NASA C-MAPSS lab

Condition Monitoring

Vibration Analysis

FFT spectral analysis identifies bearing fault frequencies: BPFO, BPFI, BSF, FTF.

Temperature Trending

EWMA smooths noise. Thresholds: Normal <50C, Warning 50-65C, Critical >65C.

Oil Analysis

Particle count, viscosity, TAN. Trends predict maintenance 50-100 hours ahead.

RUL Prediction

Kaplan-Meier Survival

S(t) = Product(1-d_i/n_i). Median RUL when S(t)=0.5.

Cox Proportional Hazards

h(t|X) = h_0(t) * exp(sum(beta_i * X_i)).

Random Survival Forest & LSTM

RSF: forest ranking. LSTM: temporal degradation sequences.

Anomaly Detection

CUSUM & EWMA detect gradual drift. Isolation Forest handles multivariate anomalies. Autoencoders flag reconstruction errors.

P-F Curve & Maintenance

Potential to functional failure interval. Cost = (failure_cost*P(fail)) + (inspection_cost*count).

Hands-On Lab: NASA C-MAPSS

100 engines, 21 sensors. Rolling features -> RSF -> C-index evaluation.

Expected: Predict failure within +/- 50 cycles.

Key Concepts

| Concept | Definition |
| --- | --- |
| BPFO/BPFI/BSF/FTF | Bearing fault frequencies |
| EWMA | Exponential weighted moving average |
| RUL | Remaining useful life |
| Kaplan-Meier | Non-parametric survival |
| Cox PH | Proportional hazards model |
| Concordance Index | RUL ranking metric |
| P-F Curve | Failure detection interval |
| CUSUM | Cumulative sum chart |

References

NASA C-MAPSS: https://ti.arc.nasa.gov/tech/dash/groups/pcoe/

Harrell, F. (2015). Regression Modeling Strategies. Springer.